TEKTERM 1.60

TEKTRONIX 4010 EMULATION FOR THE IBM-PC

BY GREG SHERMAN

PREFACE

This  reference  manual  describes the operation of the TekTERM Terminal

Emulation Program on an IBM PC or compatible computer.   In addition  to

describing  the  capabilities  of  the program and its hardware require-

ments, this documentation provides the following specific information:

o   How to set communication parameters and operate your PC as a  termi-

nal.

o   How  to connect to mainframe operating systems including DEC VAX/VMS

and IBM VM/CMS.

o   How to save graphics screen images to a disk file  and  how  to  re-

trieve these files for display at a later time.

o   How to configure TekTERM for a particular host system.

This  manual  assumes  you are familiar with the PC-DOS operating system

and that you have had experience working with a mainframe computer.

NOTE: Throughout this manual, the enter or return key (large left point-

ing arrow to the left of the 4 & 7 on the numeric keypad, <---+  on  the

standard IBM-PC keyboard) will be represented by <ENTER> or <CR>.

FILES ON YOUR TEKTERM DISKETTE

The  following files should be on your TekTERM program diskette in order

for TekTERM to operate properly:

TEK.EXE                 This is the TekTERM program file.

TTUTIL.COM              This program is the  TekTERM  graphics

screen manager.

TT.CFG                  This is the default TekTERM configura-

tion file.  This file is not required;

however,  it is recommended that it be

present with its configuration parame-

ters defined to match the  values  you

use most often.

The  following configuration files are also included on the TekTERM dis-

tribution diskette.  These files contain  the  configuration  parameters

typically used for the specified communication interfaces:

VAXMODEM.CFG            dial-in communication with VAX.

VAXLOCAL.CFG            LocalNet communication with VAX.

CMSMODEM.CFG            dial-in communication with CMS.

CMSLOCAL.CFG            LocalNet communication with CMS.

<<<<<  TEKTERM  >>>>>

INTRODUCTION

The  TekTERM package allows an IBM Personal Computer with graphics capa-

bilities to emulate Tektronix models 4010 and 4010-1 graphics terminals.

Users who are familiar with the keyboard, design, and functions of these

terminals should have little trouble adapting to TekTERM.  CMS users who

are unfamiliar with Tektronix terminals should refer to the Appendix  of

this manual for supplemental information.

TekTERM will:

o   Accept all standard Tektronix protocols for displaying graphic data.

o   Emulate the GIN (Graphic Input) functions of the Tektronix 4010.

o   Save graphic images for off-line recall.

o   Record  multiple  configurations for operation in different environ-

ments.

If you are familiar with graphics programming  on  a  mainframe  system,

you'll appreciate that TekTERM is compatible with the Tektronix PLOT-10,

AGII,  and TCS libraries, as well as with the Interactive Graphics Pack-

age (IGP).   TekTERM will also  allow  you  to  operate  most  graphics-

oriented   packages  including  PREVIEW  and  SAS/GRAPH.    FORTRAN  and

SAS/GRAPH graphics packages are described  in  Virginia  Tech  Computing

Center User's Guides GR01 and GR09.

WARNING: DO NOT INCLUDE PROC GREPLAY STATEMENTS IN SAS/GRAPH PROGRAMS TO

BE  EXECUTED  WITH  TEKTERM.   PRESENCE OF THIS STATEMENT MAY PLACE YOUR

PROGRAM INTO AN ENDLESS AND EXPENSIVE EXECUTION LOOP.   TO RECOVER  FROM

THIS LOOP, PRESS THE CTRL AND BREAK KEYS SIMULTANEOUSLY.  THEN ENTER THE

COMMAND  "HX"  TO HALT  THE EXECUTION OF YOUR SAS PROGRAM.   IF YOU ARE

DROPPED INTO CP MODE, ENTER THE COMMAND  "IPL CMS"  TO REINITIALIZE YOUR

CMS SESSION.

REQUIREMENTS

The following hardware is needed in order to use TekTERM with either the

VM/CMS or VAX/VMS systems.

o   An IBM PC, Portable PC, XT, or AT with 64 KB memory and one diskette

drive (two are recommended for picture file capture).

o   An IBM color/graphics display adapter and display.  (The amber  dis-

play of the Portable PC is acceptable).

o   An asynchronous communications adapter and a data communications de-

vice.   This device can be an internal or external modem, or a local

area network connection.

o   PC-DOS Version 2.0 or higher.

NOTE: The modem or other communication device you use must  be  able  to

operate  in  a FULL DUPLEX mode.  That means it must be able to transmit

and receive data simultaneously.  (The duplex of your modem is  NOT  re-

lated  to  the  duplex setting of the terminal or of the host computer).

Most modems are full duplex.

PREPARING A WORKING DISK

The TekTERM program diskette  is  not  copy  protected.    Before  using

TekTERM,  you  should make a working copy of your program diskette.  You

can do this by placing a new, unformatted diskette in drive B  and  your

DOS system disk in drive A.  Then type:

FORMAT B:/S  <ENTER>

and respond to the prompts.  When the formatting is complete, place your

TekTERM program diskette in drive A.  Type

COPY *.* B:  <ENTER>

to  transfer  the TekTERM files to your new diskette.  Your working pro-

gram diskette is now ready and should be used exclusively.  Your  master

disk should be stored safely.

If  your  PC  is equipped with a hard disk, it may be convenient to copy

the TekTERM files onto the hard disk.

CONFIGURING TEKTERM

The TekTERM program is easy to operate.   The main program  consists  of

two parts: configuration and terminal emulation.

CREATING CONFIGURATION FILES

Each host system or communications connection requires a specific termi-

nal  configuration.  Although TekTERM manages most of the configuration,

there are some parameters that must be set manually in order for  commu-

nication to be successful.

To  configure  TekTERM,  start  the main program by placing your working

TekTERM disk in drive A and entering:

TEK   <ENTER>

at the A> prompt.  In a moment, the main TekTERM  screen  will  be  dis-

played:

------------------------------------------------------------------------

T e k T E R M  V 1.60

Tektronix 4010 Emulation for the IBM PC

No. 0A00000

Copyright (C) 1985 Greg Sherman

Alt-S: Set Up      Alt-L: Load Config      Alt-C: Connect      F10:Exit

------------------------------------------------------------------------

Configuration File:   TT.CFG

------------------------------------------------------------------------

Note  that the default configuration file is TT.CFG.  TekTERM will auto-

matically load this configuration file if it resides in the same  direc-

tory as the TekTERM program.

To  modify  and  save  your  configurations, enter the Alt-S combination

(hold down the ALT key then press S).  The screen will  change  to  dis-

play:

------------------------------------------------------------------------

Baud        300         Default Drive         A:

Parity     NONE         Communications PortCOM1:

Data Bits     7         Gin Terminate       <cr>

Stop Bits     1         Text Color          BLUE

Duplex     FULL         Graphics Color      BLUE

Backspace Sends    <del>

------------------------------------------------------------------------

To  select a parameter, use the up and down arrow keys on the right side

of the PC keyboard.  To adjust the parameter, use the large  +  key  lo-

cated  on  the far right side of the keyboard.  These are the parameters

that can be adjusted:

BAUD RATE            The baud rate of your terminal refers to the  speed

at  which  data  is transferred between TekTERM and

the host computer.   Baud translates to  "bits  per

second" and valid values are 300, 1200, 2400, 4800,

and  9600.  Your baud rate setting is a function of

your communications  device  (i.e.  your  modem  or

LocalNet connection) and should be adjusted accord-

ingly.  Dial-in communications support 300 or 1200;

most LocalNet lines are 4800 or 9600.

PARITY               Parity is an error-checking protocol which involves

setting  a  particular bit in each character as the

PARITY BIT.  To avoid corrupted data, the parity of

TekTERM should be matched to the  host.    Virginia

Tech CMS users should set parity as EVEN.  The par-

ity checking protocol will not correct transmission

problems,  but TekTERM will inform you if a charac-

ter error has taken place.

DATA BITS            The data bit setting of your system refers  to  the

number  of  bits  that will form a word of data for

transmission.   While most  asynchronous  terminals

will  use  a  setting  of 7, certain binary systems

will look for a setting of 8.   The  Virginia  Tech

systems should use a setting of 7.

STOP BITS            The  stop  bit protocol allows both TekTERM and the

host system to recognize the start  and  end  of  a

data word.  The normal setting is 1.

DUPLEX               Duplex  refers to the ability of your host computer

to echo back information received from TekTERM.   A

full duplex system will take all of the information

it  receives  and  send it back to the terminal;  a

half duplex system will not.  When TekTERM is oper-

ating at FULL duplex, it waits for the echoed char-

acters before placing them on your screen.  When it

operates at HALF duplex, TekTERM places the charac-

ter on your screen as you type it.    (If  you  see

double characters on the screen after having logged

on,  you  probably  need  to check your duplex set-

ting).   VAX/VMS systems should  be  set  to  FULL,

VM/CMS systems should be set to HALF.

DEFAULT DRIVE        The  default  drive  setting  allows  you to direct

TekTERM screen dumps or file transfers to a partic-

ular disk drive by default.  You may  override  the

default with drive and path specifications.

COMMUNICATIONS PORT  TekTERM needs to know which port contains your com-

munications  device  (i.e.  your  internal modem or

asynchronous communications adapter).  PC-DOS  will

allow you to configure two such devices, defined as

COM1  and  COM2.    The  default TekTERM setting is

COM1.

GIN TERMINATE        The Tektronix Graphics Input (GIN)  mode  allows  a

terminal user to send graphic data to the host dur-

ing certain graphics applications.  Because TekTERM

supports  GIN mode, it needs to know what character

your host system requires to mark  the  termination

of  graphics  input.    The default setting is <cr>

(carriage return)   both VAX and  CMS  accept  this

character.

TEXT COLOR           TekTERM  allows  you  to  set the color of the text

displayed on the screen to whatever you  find  most

comfortable.

GRAPHICS COLOR       Setting  this parameter will set the color when op-

erating in graphics mode.

BACKSPACE SENDS      The two most common characters for  sending  a  de-

structive  backspace  (or rub-out) to a host system

are ASCII 8 (backspace)  and  ASCII  127  (delete).

VAX  systems accept ASCII 127 while VM/CMS will ac-

cept  any  character as the delete character.(1) To

send  an  ASCII  8,  this  configuration  parameter

should  be  set to <bs>; to send an ASCII 127, this

parameter should be set to <del>.

For CMS, the backspace is suggested.  After logging

on, enter the command CP TERM CHARDEL <-,

where <- is the backspace key.

RECORDING A CONFIGURATION FILE

Once the parameters have been set, they can be saved in a  TekTERM  con-

figuration  file.   Press the Alt-S combination and enter a file name at

the prompt.  The file name should be 8 characters or less in length  and

does not need an extension.  If you'd like to make the new configuration

the  default,  enter  TT as the name of the file at the prompt and press

ENTER.

If you would like to modify an existing configuration file, press  Alt-L

at  the  prompt  and you will be prompted for the name of the file to be

loaded.  Use the up or down cursor keys to  select  the  parameters  you

wish  to change and the large + key on the right side of the keyboard to

select the required parameter value.  To save this file under  its  ori-

ginal name, you need only press the <ENTER> key.  To save the file under

another  name,  enter the filename you wish to use (TekTERM will add the

extension CFG) and press <ENTER>.

RUNNING TEKTERM

Before you start TekTERM, boot up the computer with a  DOS  system  disk

(DOS version 2.0 or higher) in drive A. (Your working disk will function

as  a  DOS system disk as it had been formatted using the /S option; the

master disk will not.)

TekTERM should be in the A drive.   Type   TEK   <ENTER>  to  start  the

TekTERM  program.   You should keep your TekTERM program diskette in the

disk drive until you have connected with the host computer.

When you first start TekTERM, the  program  searches  drive  A  for  the

TT.CFG  configuration file.  If this file is not present on the A drive,

the configuration parameters are set to the program default  values  il-

lustrated on page 3.  If TT.CFG is present, the configuration parameters

are set to the values specified by this file.

The  TekTERM screen displays the configuration file that is currently in

effect at the bottom of the screen.  If TekTERM found  TT.CFG,  it  dis-

plays this name.  Otherwise, it displays NONE.  You can call a different

configuration file into memory by pressing the Alt-L combination and en-

tering  the  new  configuration file name.   You may modify the communi-

cations parameters by selecting Alt-S as described previously.

NOTE: You do not need to save a configuration in order for it to  be  in

effect.

CONNECTING TO THE HOST COMPUTER

When  you've  set  the correct configuration parameters, you're ready to

enter  the  TekTERM terminal operation mode(2).  Enter an Alt-C combina-

tion to connect TekTERM to the communications device:

o   If you've got an internal autodial modem, enter your modem

dialing commands.

o   If you're using an external or acoustically coupled modem,

dial and connect it now.

o   If you're connected to a local area  network,  the  system

prompt (#) should appear.

You  should  now  be connected to the host system.  IF YOU DON'T GET ANY

RESPONSE, CHECK THAT YOU'VE USED THE CORRECT  COMMUNICATION  PARAMETERS,

particularly  the  BAUD RATE.  You can return to the TekTERM main screen

by pressing F10 and EXIT from TekTERM by pressing F10 a second time.

OPERATING AS A TERMINAL

Once you've connected to the host system, TekTERM should be invisible to

you.  CMS users who are unfamiliar with Tektronix terminals should refer

to Appendix A of this manual for supplemental  information;  VAX users

should refer to Appendix B.  TekTERM will automatically switch into

graphics mode when necessary and requires no additional set-up.

TekTERM  places a status line at the bottom of the display to inform you

about communication errors and program settings.   It  does  not  appear

while TekTERM is in graphics mode.

----------------

Note:  TekTERM supports CMS in line mode (VM1 or VM2);   full-screen

(VM1E or VM2E) editing features are not available.

----------------

SPECIAL KEYS IN TEKTERM

FUNCTION KEY DEFINITIONS

The  ten  function keys on the left side of the PC keyboard have special

functions in TekTERM terminal operation mode.  They are defined as  fol-

lows:

F1 PAGE                  clears  the  screen  and  returns the screen to

text mode.  This key is equivalent to the  Page

or  Reset  key on the Tektronix 4010 and 4010-1

keyboard.

F2 HELP                  displays the function key definitions.  Any in-

formation currently on the  screen  is  stored.

The  F2 key should not be pressed during trans-

mission from the host as data may  be  lost  or

corrupted.

F3                       no function in this version of TekTERM.

F4                       no function in this version of TekTERM.

F5 SNOOP MODE TOGGLE     is  useful  for  debugging  graphics  programs.

While  snoop  mode  is  in  effect,  all   non-

printable  characters  (ASCII 31 and below) are

replaced by an equivalent mnemonic and will not

produce their normal effects.

F6 BACKSPACE DEFINE      toggles the definition of the backspace key  on

the  PC  keyboard between ASCII 8 and ASCII 127

without returning to the set up screen.

F7 PICTURE FILE CAPTURE  invokes the picture file  capture  feature  for

saving graphic images to a disk.  The operation

of this key is discussed in Chapter 5.

F8 NO SCROLL             sends  an  XOFF  character to the host and data

transmission is paused.  Pressing F8 again will

send an XON  to  the  host  and  resume  trans-

mission.    TekTERM will display "NO SCROLL" on

the status line to warn you  that  transmission

has been interrupted.

NOTE: The  Ctrl-S/Ctrl-Q  combination  performs

the same function as the no scroll key and will

also display the "NO SCROLL" status message.

F9 VT100                 no function in Version 1.6 of TekTERM.  See the

READ.ME file on the distribution disk  for  in-

formation  regarding  an  enhanced  version  of

TekTERM which enables this key.

F10 BYE                  returns you to the TekTERM main  screen.    The

F10  key  WILL NOT disconnect you from the host

computer nor end your host session.

TEKTRONIX KEY EQUIVALENTS

The Tektronix 4010 keyboard has a number of non-alphanumeric  keys  that

are  not found on an IBM PC keyboard.  However, TekTERM provides several

key  definitions  that  are  functionally  equivalent  to  the   missing

Tektronix keys.  They are defined as:

PAGE           The F1 function key is equivalent to the Page key.

RUB OUT        When  the  backspace key is defined as <del> (ASCII 127),

it performs the same function as Rub Out.

BREAK          The  break  function  is  simulated by the Ctrl-Break key

combination.  When Ctrl-Break is pressed, TekTERM sends a

break signal to the host lasting approximately  1/2  sec-

ond.    After  the signal has been sent, TekTERM sounds a

tone to verify the break.  For CMS users, the break func-

tion will temporary interrupt execution of an application

program.  Enter HX (Halt eXecution) to terminate the  ap-

plication.    If  you are dropped into CP mode, enter IPL

CMS to reinitialize your CMS session.  VAX users have  no

need for the break key as Ctrl-Y performs a similar func-

tion.

LINE FEED      The function of the Line Feed key can be simulated with a

Cntl-J combination.

RESET          The F1 key performs a function similar to  the  Tektronix

Reset key.

ALT MODE       The  Alt  Mode  key  on  the Tektronix 4010 transmits the

ASCII code for a right bracket (]).  The function of  the

key may be simulated by pressing the ] key.

THUMB WHEELS   The functions of cross-hair cursor control wheels located

on the right side of the Tektronix terminal is duplicated

by  the  arrow keys on the right side of the PC keyboard.

Pressing the arrow keys alone will move the cursor in  10

pixel increments; in combination with a shift key, press-

ing an arrow key causes the cursor to move 1 pixel.

PICTURE FILE CAPTURE

The  F7  key  on  the PC keyboard will allow you to save a graphic image

from the display screen to a disk file.   The utility  program,  TTUTIL,

allows  you  to recall and re-display an recorded image while TekTERM is

off-line.

When you select F7 (Picture File Capture), the graphic image will momen-

tarily disappear.  TekTERM displays

PICTURE FILE NAME:

Enter a file name of eight characters or less.   TekTERM  will  automat-

ically  create  a file on the default drive with an extension of ".PIC".

When the picture file has been saved, TekTERM displays

TAP ANY KEY TO CONTINUE . . .

if no errors have occurred.

TTUTIL - THE PICTURE FILE MANAGER

The TTUTIL program is included on the TekTERM diskette to allow  you  to

review  and  print  picture  files  that  you've saved during a previous

TekTERM session.  TTUTIL is a separate command issued from  DOS;  it  is

not issued while executing the TekTERM terminal emulation program.

To invoke TTUTIL, enter TTUTIL  <ENTER>.  When TTUTIL is called, it dis-

plays

PICTURE FILE NAME:

Enter  the name of the picture file that you want to see.  You only need

to specify the filename (eight characters or less); TTUTIL will add  the

extension  and  drive if needed.  If the corresponding "picture" file is

present, it will be recalled and displayed.  The plot will be  presented

as a white foreground against black background;  no other color combina-

tions  are  available  in  TTUTIL.    Tap any key to return to the first

TTUTIL screen.

You can exit TTUTIL by pressing [ENTER] in response to the picture  file

prompt.

If  you  have  a  graphics  printer, TTUTIL can also be used to obtain a

hardcopy of the screen image.  Before executing the TTUTIL command,  en-

ter  the  command  GRAPHICS from DOS.   Then, after the desired image is

displayed on the screen, depress the SHIFT and  PrtScr  keys  simultane-

ously.    The screen image will then be printed sideways on your printer

to fill an 8.5 X 11 inch sheet of paper.

------------------------------------------------------------------------

APPENDIX A:  USING TEKTERM WITH CMS

The following appendix is included as an aid to CMS users who may not be

familiar with using a graphics terminal.  The TekTERM  program  emulates

(makes  the  PC  behave  as if it were) a Tektronix 4010 terminal.  Thus

your PC will now share the properties of this terminal.  In addition  to

being  able to execute graphics programs, you will be able to create and

modify files using line mode editing commands.

SPECIFYING YOUR DEFAULT CONFIGURATION FILE

The TekTERM distribution diskette contains  4  configuration  files  de-

scribed  in  the  Preface.    If  you  are using LocalNet, copy the file

CMSLOCAL.CFG as TT.CFG;  if you are using a dial-in phone line, copy the

file CMSMODEM.CFG as TT.CFG and check that its baud rate matches that of

your modem using the ALT-S  option  from  the  first  TekTERM  selection

screen.

LOGGING ON

Execute  the TekTERM program by typing TEK.  The f#ction

screen.

LOGG