The XDIR Directory Program

XDIR  (Extended Directory Display) is an enhanced version of

the  Digital Research 'built-in' CP/M command 'DIR' which  allows

the  user to  search  a range of drives and/or user  areas for  a

specified   file,  and create a sorted  directory  display.  This

capability is patterned after FILEFIND, but is significantly more

useful in that  XDIR will report the sizes of files matched.    A

new  BDOS intercept  routine  has been added, which although more

involved   than   the  FILEFIND  method,    is   more   generally

applicable  and should  run "as-is" on most CP/M systems.   Thus,

by using XDIR,  the use of a separate FILEFIND program should  be

unnecessary.

XDIR - Basic User Operation

You  merely   type   "XDIR",    followed  by   an   optional

ambiguous   or  unambiguous  filename (as  you  would  with   the

CCP  directory command).   Omitting  the  filename  (i.e.,  "XDIR

[CR]"),  will be interpreted  as  "XDIR *.*".   Similarly,  "XDIR

A:" would  imply  "XDIR A:*.*" and "XDIR B:"  would  imply  "XDIR

B:*.*".   In  addition  to the filename option,  up to 8  command

line  options  may  be  specified.    If   options   are   to  be

specified, then  the  filename  must  be EXPLICITLY specified  by

at  least a drive code.  Option fields are  preceded  by a single

dollar  sign  as  with  most  Digital Research programs  such  as

MAC,  and may be essentially free-form in format.  Example:  XDIR

A:  $s  a  D  rn u0 fp.  Note that the reference  to the A drives

in  "A:" is mandatory.     Invoking  XDIR with a command  of  the

form  "XDIR  $s a D rn u0 fp" would result in XDIR thinking  that

$S  was the file to be searched for.   Note also that spaces  and

case   within   the  option field  are  insignificant  (i.e.,  "$

SADrNu0F p" is equivalent  to  "$  S A D R N U0 F P").  The  only

exception  concerns the "USER" option  which  will  be  explained

later.   Users  should also be aware of two restrictions  on  the

command  line  format.   XDIR scans the entire command line for a

dollar  sign  preceded  by  at  least  1  blank  to  delimit  the

beginning   of  the option field.   For  this  reason,   invoking

XDIR  with a command line  such as "XDIR $ $AR" is illegal  since

XDIR  will consider the first dollar sign to be the option  field

delimiter.   If  you  need to specify  a  a filename whose  first

character is a dollar sign, precede it with  a  drive  code as in

"XDIR A:$ $AR".   In  this  instance,   XDIR  will  consider  the

first   dollar   sign insignificant as it is not  preceded  by  a

blank.

The  second restriction is that command  line options may be

specified once and only once per invocation.    That  is,   "XDIR

A:  $XDIRS"   would be illegal since the "S" option is  specified

twice.   This shortcoming  is  due  to  the fact that options can

be made into defaults at assembly time.

The eight options recognized are:

"S"  - system  option :  system files will be included  in  the

output rather than being suppressed.

"F"  -  file  option :  the directory  output  will  be  echoed

to   a   disk  file  named  "XDIR.DIR"  on  the  DEFAULT

drive.  If XDIR.DIR already  exists,  then the directory

output  will  be  APPENDED  to the  end  of   the  file.

Otherwise,  XDIR.DIR will be created as a new file.  The

append feature allows you to build  up  one massive file

on  the default drive containing the directories of  all

of  your  disks,  without  having   to  concantenate   a

swarm  of individual directory  files.

"U"  - user option :  allows  the  specification  of  the  user

number  for  the directory of the form "Uxx"  where  the

user   #   is  greater  than 0,  but not greater than  a

specified  value  not to exceed  15.   The  user  option

specification  will  be illegal if the user # is out  of

range,  or omitted.  Note  that since  the  user  option

requires   a  parameter (i.e.,  the user  number),   the

user   number must immediately follow the "U" option  on

the command field.    That is, "U1" is legal, but "U  1"

is not.   In this respect, the U option differs from the

other   options    in    that    embedded   spaces   ARE

significant.

"A"  - all users :  causes XDIR to display  directories of  all

user   areas   starting  at  the  user   area  specified

in   the   U  option or,  if U option  is  omitted,  the

default  user  area  and continuing up  to  the  maximum

number of user directories (15).

"R" - reset option : allows for automatic resetting of the disk

prior  to  performing  directory  search  so  that   the

allocation vector  will  be  updated.   Same as doing  a

CTRL-C  when  changing disks,  but handy if  you  didn't

(such  as  when  running  a SUBMIT  file).   Useful  for

XDIRing a stack of disks with the "F" option, but  slows

down XDIR considerably in normal use.

"N"  - no  page option :  unconditionally  disables   the  page

pause   option.    Useful  when running XDIR in the  "F"

and/or  "A"  modes when you don't want the  page  prompt

slowing you  down.    Note that XDIR will  not  put  the

page-pause prompt  into  the output file.

"P" -  printer option : forces all console output to be  echoed

to  the CP/M  list  device,   with the most  significant

bit set to 0.

"D" - all disk option : allows XDIR to search  all  disk drives

on-line   starting  with  the  disk drive specified   or

implied   with   the   command   line  filename.     For

example, "XDIR B: $D" will result in XDIR searching  all

drives  beginning  with  drive B.   "XDIR *.*  $D"  will

result  in XDIR searching all drives beginning with  the

drive  that is logged in.

If   an  unrecognized  option  (or  illegal   user    option

specification) is detected,  then the command line will be played

back  to the  console  up  to  the  point  where  the  error  was

detected.