BOOT ROM IMPLEMENTATION NOTES

This document is for maintainers and developers of the Victor/Vicki Boot

Roms.  Information consists of an explanation of the design philosophy of

the boot process, notes on building and debugging the boot rom, and

an overview of the boot rom's functionality and architecture.

The boot rom is located at a fixed address in memory.  This area is not

part of regular memory, but is "hard-coded" in read-only memory.  The

area dedicated to the Boot Rom is from FE000 to FFFFF (8K bytes).  There

may be talk about the boot going from FC000 to FFFFF (16K bytes).  The

area from FC000 to FDFFF is a second image of FE000 to FFFFF for certain

versions of the CPU board, and available in others.  Since this area is not

available for all CPU's (and there is no way to get 16K bytes of ROM on

the board), the boot ROM really is limited to its 8K bytes.  When

a reset occurs (also on power-on), hardware automatically vectors to

the boot rom's code.  The rom is then responsible for loading in the

operating system (from floppy, network, hard disk, or diagnostic load

from the serial port), and performing whatever I/O is necessary to load

the O/S.  Since I/O and other errors may occur, it is also important that

the boot rom provide reasonable diagnostics so that these errors may be

corrected (since potentially no software can be loaded, the rom is on its

own until any I/O errors are corrected).

1.  Philosophy

The boot rom for the Victor 9000 and Vicki is responsible for loading in the

Operating System AND NOTHING MORE.  It is important that a minimum amount of

hardware is utilized during the boot process.  Because the boot must work to

even load diagnostics--timers, interrupts, etc. are not used during the boot

process (it's just one more thing to fail and prevent diagnosis of the pro-

blem).  The philosophy is to ensure a good chance of loading software (not

necessarily optimizing performance, and certainly not exercising the machine).

A second effect of doing only what is required to load software is that

the state of the machine following the boot rom's execution shall be indet-

erminate.  The boot rom will not engage in system initialization (this is

left to the operating system).  There is little enough room in the ROM

for this type of set-up, plus we run the risk of hanging the ROM every time

additional hardware is accessed.  Only hardware which is required should

be dealt with.

One exception to this is certain Diagnostic and Field Service routines.

Because the boot rom is the only way to get software to execute, special

code for loading diagnostics during manufacturing test and field problem-

isolation is included.  Also Field Service has some code added to aid in

detection of the cause of a boot failure and to provide a list of recov-

erable errors (failures which did not stop the boot process, but may be

indicative of later problems), which are available to Field Service person-

nel for investigation.

2.  Building and Debugging

Development

To conserve space (the ROM is only 8k bytes), the boot rom is written in

assembly language.  Batch files exist for building, archiving, and outputting

listing files during the development process.  All boot files are in the

format xxxyyyyy.zzz where xxx is an identifier of which boot product the file

belongs to (at this time, there are two boot products, VCK--the Vicki boot

rom, and BT1--the universal boot rom for the V9000).  The Vicki product has

differences in only some of the files, and uses BT1 files commonly with the

Victor 9000 boot.  The important differences are in the xxxINIT module

(different version numbers), xxxBASE (different settings for CRT contrast),

and the xxxFDDVC module (totally different way to do I/O and control the

floppy drives).  Batch files are:  xxxSAVE.BAT, xxxLIST.BAT, and

xxxBUILD.BAT .  The save job will copy all "important" files for the xxx

boot product to a formatted, scratch floppy disk.  The list job will copy

all .LST files for the xxx product to a diskette with your favorite list

programs on it.  The build job will build (assemble and link) the boot rom

for the xxx product.

Debug

Use MS-DOS 2.0 in the development of the boot rom (I cannot vouch for debug

techniques in other environments).

The DEBUG package may be used effectively to debug and validate most of the

boot rom's code.  Once the .EXE's are produced, they may be tested (separ-

ately).

For testing the xxxINIT.EXE module, many of the segments of code

must be skipped over (or else they'll clobber your O/S).  Skip over the

instruction to "shut down the CRT" (otherwise you're in the dark).  The

checksum code may be executed under control (to check if it's accessing

what it's coded to).  The check for the diagnostic rom at D0000 should be

operable (don't be concerned if it appears that ROM is present if your machine

doesn't have that much memory--not having a JMP instruction will make the

test fail).  Testing screen ram is fine if you don't try to stop half way

through (this will work, but it's an ugly sight to behold).  Testing the

first 16K can be tested by substituting an address (in ES:) exceeding your

debugged program (so it's out of the way).  This is all you can do to test

the INIT module.  Stay away from the code to test for non-fatal errors in

the PIC, keyboard, etc. ports--it'll clobber your system.  This code may

only be tested after a ROM is built.

For testing the BT1BOOT.EXE module, a much more comprehensive debugging

can be done.  There are still areas to watch out for, however.  All of the

following tricks and things to look out for are in the BT1BASE module.

Don't worry about the "overscore" cursor, when ROMs are built, it will

go away (this is caused by interaction with DEBUG and the boot code).  Once

in DEBUG, and before executing the module, set 0000:0 to a 1 and 0000:1 to a

zero (this will bypass the boot's memory test--and allow entry of the pro-

gram).  Also, the word at 0:300 must contain the segment (remember to

reverse the bytes) of the end of memory.  This is used by the boot to

determine where to load the Operating System.  Memory looks like this:

Real End of Memory (say segment 4000 on a 256K machine)

Operating System

.......empty space.........

BT1BOOT (program being DEBUGGED)

DEBUG.COM

Low Memory Junk

The trick is to set the end of memory to an area below (less than) the

start of the operating system.  The boot will load the O/S in what it

believes to be high memory (that is, below the Operating System you're

using to DEBUG).  If "empty space" is big enough to hold the new O/S,

you're in fat city (if not, add more memory to your machine).  Following

testing the boot (if it works) you'll be executing the new (lower) O/S.

This can't go on forever, so it's a good idea to RESET after each O/S is

successfully loaded (for a failure, better RESET, because you may have

trashed the O/S anyway).  So set 0:300 to 0 and 0:301 to the high order

byte of the segment address below your operating system.  Remember that

if the O/S doesn't fit, you'll crash your program (the boot checks if

the O/S fits in the area from High-Memory to 0--not from High-Memory to

the end of your debug session).  For the VICKI, be sure to skip over

the code which sets the screen contrast/brightness (it will turn your

screen off on a V9000).

To test the memory-test logic, instead of the above, have debug execute

the memory test logic, but start at the beginning of your "empty area"

instead of 16k, and end at the end of the "empty area" (instead of 896k).

This can be done once to see if the memory test/size code is reasonable.

This module can be executed "as-is" (with the above preparation).  The

character set font is positioned as to be added to an inconsequential area

of the existing font (so debug is operable).

You may now freely debug the boot code (better write-protect any floppies

you're using and be prepared to clobber any hard disk data you have).

One last caution.  During the load of the last sector of the O/S, the CRT

is turned off (this is because the font area is used as a buffer).  You may

either set a break-point after the CRT is back on, or skip over the code

which turns the CRT off.

Burning ROMs

Once you've debugged as much as you can, ROMs need to be created.  Use the

program SYSLOC (from the Programmer's Toolkit):

SYSLOC xxxBOOT  .BIN  /S

SYSLOC xxxINIT  .BIN  /S

to create a .BIN file from the .EXE (create a binary image of the code).

If either SYSLOC gets an error, dump the .EXE file's contents to find the

relocatable code which you've added.  Change the addressing mode that

you're using so that the .EXE file will be relocatable respective to its

segment definitions.

Now, we'll create a ROM image.  Run DEBUG (with no parameters), and issue

the following commands:

-fcs:1000 L2000 0                   to fill memory with zeroes

-nxxxboot.bin                       the base boot program and drivers

-l1000                              load the main program at cs:1000

-nxxxinit.bin                       the boot's initial entry point

-l2D00                              is at 1D00 more (the xxxINIT module's

starting address, which was originally

based upon its size)

(Currently FFD0:0 minus FE00:0)

the boot image is now at cs:1000 to cs:2fff

-rcx <- 1000                        will checksum 1000 words

-rsi <- 1000                        starting at cs:1000

-rax <- 0                           initialize checksum accumulator

-ecs:500 <- 2e 03 04 46 46 e2 f9    program to compute the checksum

-rip <- 500

-g507                               execute the checksummer

the boot image's checksum is in AX

compute 2152 hex minus AX (you can use "-h2152 xxxx")

-ecs:2ffc <- low byte  high byte    store the checksum

checksum is stored in the image

-rbx <- 0                           set high-order of file's length

-rcx <- 2000                        low-order length in bytes

-nxxximag.bin                       output file name

-w1000                              write out the image from cs:1000

By the way, always use a separate checksum routine.  If you use the ROM

code's checksum routine, DEBUG will put the INT 3 instruction in the code

you're checksumming, producing an error in one of the bytes by the sub-

stitution of the CC hex opcode.

You now have in the file xxximag.bin the ROM image.  You should run DEBUG

again to compute the checksum (as above).  The checksum program will probably

still be in memory.  The resulting checksum the second time should be (in AX)

2152 hex.  If not, try again, with all those numbers you've probably made a

mistake, and better finding it now than after the ROM's made.  By the way,

the checksum value was chosen because empty ROM or malfunctioning ROM's may

sum to FFFF or 0000 the value 2152 is NOT(DEAD hex).

Run the program BINHEX (on this disk) to produce a hex source listing of the

ROM which is suitable for down-loading to the DATAIO ROM-BURNER.

BINHEX xxximag.bin   xxximag.hex

-----------------------------------------------------------------------------

FOR FINAL VERSION:	the document control people require a hex listing

of the ROM for release of the EC.  To do this, issue the following commands--

DEBUG > HEXLIST			debug with output redirected to a file

Nxxximag.bin			you won't see this (it's sent to disk)

L1000				load the hex image (it's ASCII)

dcs:1000 l2000			prints the ROM's values to the file

q				(it will take a while)

The file HEXLIST can now be manipulated with PMATE (from the programmer's

toolkit).  Use an old rom hex listing for the format.  Primarily, the segment

addresses must be changed (use global changes/macros for this), headings must

be added, and the file split into two (one for the FE00:0 rom, and one for the

FF00:0 rom).  For Vicki, since the rom split-up is even/odd bytes rather than

splitting it in half, you've got some additional PMATE work to do at the start

(have fun).  An alternative approach is to install the new boot rom in your

machine and do a DEBUG dump (so the addresses won't have to be changed).

------------------------------------------------------------------------------

Now, trundle off to the lab with xxximag.hex, term (terminal emulator program

from the library), and an operating system on a disk.  You need the DATAIO,

hooked by a serial cable to a V9000, 2 clean 2732 prom's, and your disk.

Run term on the V9000.  Select a baud rate of 4800, and the correct port.

Select a file name (alt-A, alt-B, f) of xxximag.hex .  At the DATAIO,

turn it on (wait for self-test to finish), press:

SELECT

83

START				enters the 83

START

It will display Intel MDS format (or something like that).

COPY				copy from

PORT				I/O port (serial)

0				address 0 (byte 0 of the serial stream)

START

2000				for a length of 2000 bytes

START

RAM				copy to the DATAIO's memory

0				at address 0

START

The DATAIO will display INPUT PORT and a clock, when the hands move,

it's receiving.  Go to term and type the transmit command (ALT-A, ALT-B,

t).  The file will be sent (you've got 5 to 10 minutes.

-------------------------------------------------------------------------

If this is for VICKI, you've got to split odd and even bytes:

SELECT

A5				splitting function

1000				symmetric to address 1000 (0 to 2000)

START

It will display SPLIT and the clock, then **.

------------------------------------------------------------------------

Put the ROM labelled FE (V9000) or EVEN (VICKI) in.

COPY

RAM				copy from DATAIO memory

0000				address 0

START

1000				length of 1000 (half per ROM)

START

DEVICE				to ROM device

0000				start at offset 0 in ROM

START

1924				description of 2732 ROM

START

Now, you've got to wait while the PROM is programmed.  When completed,

insert the second rom (labelled FF or ODD).  Use the same copy sequence

(above), but start at address 1000 instead of 0000 (the 0000 directly after

the RAM is changed to 1000).

Now you'll probably have to use the ISIS and ICE-86/88 to debug in a real

environment.

3.  Functionality and Architecture

For your information, also submitted to the library with this product are

definition of the Omninet protocol, and hard disk label format.

The following are the source files for the boot roms:

File Name	Description

------------------------------------------------------------------------------

xxxTTL  .INC	title and legal disclaimer to be "included" in all sources

xxxINIT .ASM	initial power-on/reset code, this module is presently "hard-

located" at FFF0:0 (and may be up to 100hex bytes long), it

is linked by itself as a single module

xxxBASE .ASM	module following power-on/reset, main driver for boot process,

this module must be the first one linked with all other modules

(except xxxINIT) and is "hard-located" at FE00:0 (and may be

2000hex minus 100hex long)

xxxCHINT.ASM	initializes the character font

xxxCUOPN.ASM	dispatches control unit operations, calls I/O drivers to

process the "logical access" requests it receives

xxxNTDVC.ASM	network device handler, does all network I/O

xxxHDDVC.ASM	hard disk device handler, does all hard disk I/O

xxxFDDVC.ASM	floppy disk device handler, does all floppy disk I/O

xxxVARS .ASM	variable definitions, these are located starting at 0000:0

since the ROM can't be written to (this area is not included

in the length of the boot rom's code)

xxxICONC.ASM	icon character font definitions, bit-mapped icons for the CRT

xxxBVT  .STR	structure definition for the "Boot Vector Table," this is an

area in low memory (0:300) used for Software (O/S, Diagnostic)

communications

xxxUL   .STR	structure definition for the "Unit List," this is a table

of various control units and their characteristics used by

the xxxBASE.ASM module to select control units (devices) to

boot from

xxxLRB  .STR	structure definition for the "Load Request Block," this is

a parameter list created by xxxBASE.ASM, passed to

xxxCUOPN.ASM, and to the specific device driver describing

a generic logical boot-related operation to perform

xxxICONS.INC	include file of equates for:  the character set indexes of

each icon, and icon positions on the CRT

xxxFDLBL.STR	structure definition of the floppy disk's label

xxxNTLBL.STR	structure definition of the network server's label

xxxNTCTL.STR	structure definition of network control protocol

xxxHDLBL.STR	structure definition of the hard disk's label

------------------------------------------------------------------------------

A brief explanation of the boot sequence is now given.  This is not intended

as an implementations specification.  It will introduce you to the boot and

allow you to more easily understand the code.  I hate to say this, but there

really is good documentation in the listings' comments.

Upon reset or power-on, the module xxxINIT.ASM is given control.  This module

gets comfortable in the machine (set the NMI interrupt vector, turns off the

CRT), and sets the BOOT OPTION FLAG to zeroes (that is, memory will be tested

and the loaded program (O/S) will be executed).  The ROM's checksum is verified,

and the diagnostic ROM at D000:000 is checked for.  The boot supports the cap-

ability for a ROM to exist at D000:0 the ROM must start with a JMP instruction,

E9.  If it is present, the boot will vector to that ROM (and let the diagnostic

do what they will).  The first 16K of memory is verified (except for the first

100hex bytes, which is a save area for software).  Screen Ram is tested, non-

fatal errors are checked for in the PIC, keyboard, parallel, and user ports; and

the xxxINIT module jumps to xxxBASE for the boot.

xxxBASE is passed control by xxxINIT (in which case the boot option flags are

zeroed) or may be jumped to directly by software (with interrupts disabled).

See below for Software usage of xxxBASE.  xxxBASE sets up interrupt vectors,

the stack, copies the table of units to boot from to RAM (since it will be

written into), and sets up the CRT screen.  The floppy LED's are blinked to

indicate to the outside world that we're alive.  All load units are polled

for their existance (this eliminates trying to later boot from devices which

are not present, and allows a single call for set-up of the devices).  A

loop is then entered which tries to boot the machine.  The sequence of this

loop is:

1.  call all I/O drivers to reset them

2.  test and size memory (if the flag does not indicate to bypass this)

3.  loop to find a device which is ready to boot from--

a.  check the serial port for a diagnostic boot

(if it is present, boot from that, otherwise continue)

b.  check any other units which are present (ie, attached)

if they are ready to boot, in the order left floppy,

right floppy, network, hard disk (for floppies, wait

a while to allow insertion on V9000; for floppies on

VICKI, the user must reset the machine after insertion).

If a device is ready, go to set 4 otherwise repeat step 3.

4.  put the device on-line (that is, read its label)

5.  verify that the system to load will fit in memory

6.  load the system (read the last sector into a buffer at dot_ram

so that a partial last sector will not exceed memory bounds

and write on memory mapped to I/O)

7.  execute the software

External Interfaces to the BOOT ROM

Refer to the Operator's Reference Guide publication for information on the

operator's interactions with the Boot Rom.

Reset/Power-On Vector		FFFF:0 to 4 is the JMP instruction which hard-

ware executes on a system reset or power-up.

This JMP enters the xxxINIT.ASM module.

xxxBASE Entry Point		FFFF:5 to 9 is the JMP instruction which enters

the xxxBASE module.  This vector may be used

by software which has set the BOOT OPTION FLAG

to perform loading functions (but not a real-

live boot).  For example, the diagnostic ROM

at D000:0 may call this routine when it wants

to load software, after it has determined that

the computer is operational.  Interrupts must

be disabled before jumping to this address !!!

NOTE:  when this entry is used, the flags at

0000:00 and in the Boot Vector Table (below)

must be properly set by the caller !!!!!

AREA AT 0000:00 TO 0000:FF	This area of memory is reserved by the boot.

It is not tested or set to zero.  For this

reason, any data stored in this area by soft-

ware will remain intact between boots (unless

power is lost).  This is the area in which

MS-DOS 2.0 saves the date and time, for

example.  The BOOT OPTION FLAG and Saving

of Program's Registers areas, below, are

already spoken for, however.

BOOT OPTION FLAG		The word is at location 0000:0.  The

word consists of 2 single-byte flags:

0000:0--if this byte is zero, the boot module

will perform normal memory testing and sizing.

If this byte is non-zero, memory will not be

tested and the memory size used by the boot

will be defined by the BOOT VECTOR TABLE.

0000:1--if this byte is zero, the boot module

will pass control to (jump to) the loaded

software, as is normal.  If this byte is non-

zero, upon completion of the software load,

the boot will perform an INT 255 (vectoring

to the software whose address is in SW_ENTRY,

below.  The software which is then

given control may look into the Load Request

Block (below) to ascertain the loaded O/S's

entry point, etc.

SW_ENTRY			set this double word (at 0000:2) to your

entry address if you set 0:1 to non-zero.

These flags are only pertinent when software

vectors to the boot xxxBASE module (that is

through a jump to FFFF:5), and not on a reset

or power-up.

Save of Program's Registers	When a reset/power-on boot occurs, the

registers are saved in low memory (in case

a program was running which needs debugging):

SP--at 0000:7E

AX--at 0000:7C

BX--at 0000:7A

CX--at 0000:78

DX--at 0000:76

SI--at 0000:74

DI--at 0000:72

BP--at 0000:70

BOOT VECTOR TABLE (BVT)		The BVT is located at 0:300 and is used

for the boot ROM to pass information to

the loaded software.  The following items

are in the BVT.

BVT Memsz			This is the size of memory, in paragraphs.

This word, at 0:300, can be used by software

following the boot load process (the O/S)

as the segment address of the end of memory

plus one.  This word may also be used in

conjunction with the BOOT OPTIONS FLAG word

for software to tell the boot ROM what to

use as the end of usable memory.

BVT Btdrv			This word, at 0:302, identifies to the loaded

software which device was used to load it:

High Order Byte = 0 left floppy

1 right floppy

2 network

3 hard disk

Low Order Byte  = 00hex left floppy

01hex right floppy

10hex hard disk,controller 1

20 to 70, controllers 2 to 7

80hex network, server 0

81 to 89, server 1 to 9

The above values are an index to the boot

device (high order), and an encoding of the

driver (low order byte, most significant

nibble), and the particular device (low

order byte, least significant nibble).

BVT Dvclst, Dvcccbs		These are two long pointers reserved for

future usage by the Operating System as

a base pointer for a list of devices,

and a list of device control blocks,

respectively.  Dvclst is at 0:304, Dvcccbs

is at 0:308.

BVT nfatals			This word flags any non-fatal errors found

by the INIT module:

bits 15 to 12 set to 1--the PIC

bits 11 to 08 set to 2--the parallel port

bits 07 to 04 set to 3--the keyboard

bits 03 to 00 set to 4--the user port

This word, 0:30C,D should be zero if xxxBASE

is directly jumped to by a program.

LOAD REQUEST BLOCK (LRB)	The LRB immediately follows the BVT, at

address 0:30E.  This structure is the

request block that was used to request the

I/O driver to load the Operating System.

Some information in it may be helpful to the

loaded software.

LRB Reserved			0:30E and 0:30F are reserved by the Boot Rom.

LRB Device/Unit			0:310,1 WORD of the load device's controller

and unit number:

Bits 15 to 12 = Driver's Index

0-floppy

1-hard disk

2-network

Bits 11 to 8  = Reserved for Future Use

Bits 7 to 4   = Control Unit Number

(0 for present devices)

Bits 3 to 0   = Device's Number

0/1 for left/right floppy

0/1 for hard disk controller

0 to 9 for server's number

LRB Reserved			0:312 to 0:31E reserved for Boot ROM usage.

LRB Load Address		The word at 0:31F is the paragraphs at which

the software was loaded.

LRB Reserved			0:321 and 0:322 are reserved by the Boot ROM.

LRB Load Entry			The long pointer at 0:323 is the entry address

for the loaded software.

Error codes for Field Service	When unrecoverable I/O errors are detected

by the boot ROM, and the screen is not

operational, the boot rom sends information

out of I/O port FFFFhex:

01--unreproducable error in screen ram

(when we tried to isolate the failing

bit, the test passed)

02--bad checksum in boot ROM

03--unreproducable error in first 16K of

memory

04--internal (programming, CPU) error

1x--single-bit error in screen ram,

x is the failing bits' number (0-F)

where F is the most significant bit

2x--multiple-bit error in screen ram,

x is one of the failing bits' numbers

(0-F) where F is the most significant

bit.  x is the first erroneous bit

found starting at the most significant

bit

3x--single-bit error in first 16K of memory

4x--multiple-bit error in first 16K

For all FE errors, output port FFFF hex

is written to with the single-byte error

code, in a loop.  For memory errors, the

loop looks like this:

LOOP:;

OUT	0FFFFh,error_code;

MOV	WORD PTR location,pattern;

MOV	BX,WORD PTR location;

JMP	LOOP

Where location is the failing memory

location.

Version				FFFF:A,B is the two-byte version number of

the ROM (remember that FFFF:A is the least

significant byte, FFFF:B the most significant

byte.  Possible values are:

AAAA--for floppy-only ROMs

01FF--for floppy/hard disk ROMs

F1F1--for the diskless NetWork Station

F2F2--for the VICKI universal ROM

F3F3--for the V9000 universal ROM

Checksum			FFFF:C,D is the checksum fudge word.  The

ROM's checksum is computed as an addition

of all the words in the ROM (FE00:0 to FFFF:F).

This total should equal 2152 hex (NOT DEADhex).

IBM PC Compatibility Flag	FFFF:E,F are FFhex since some applications

check this word to see if they are running

on an IBM PC or XT (we say we're an XT).

Diagnostic ROM at D000:0	If a ROM is to take control upon power-on/

reset, it may be plugged in at address D000:0

and must start with a JMP instruction, opcode

E9.  This ROM will be jumped to (at address

D000:0) following a power-on or reset immed-

iately after the boot ROM saves the program's

registers, and validates its checksum.

Serial Boot Capabilities	If the serial port A's control lines have

RING INDICATOR, DATA CARRIER DETECT, and

DATA SET READY, then 512 bytes will be loaded

from that port at 1200 baud, placed into

memory at 0:4000 and jumped to.